ModeSWeatherData

Program Design Document

Package: adsb

Class: ModeSWeatherData

Language: Java

# 1. Overview

This program is a batch process that derives meteorological values — outside air temperature, wind direction, and wind speed — from ADS-B (Automatic Dependent Surveillance-Broadcast) reception data and writes the results to a CSV file.

It reads Mode S data files collected by a Raspberry Pi or similar device, filters records to a defined airspace, and computes weather parameters only for valid observations.

## 1.1 Processing Flow

| Step | Description |
| --- | --- |
| 1 | Generate input and output file names from the current date/time |
| 2 | Read the input file (Mode S data) line by line |
| 3 | Split each line by comma and apply the airspace filter |
| 4 | Compute meteorological values for records that pass all filters |
| 5 | Write results to the output CSV file |

# 2. File Specifications

## 2.1 Input File

| Item | Details |
| --- | --- |
| File path | \\192.168.1.zz\pi\modeSdata\{yyyyMMddHH}modeS_data.txt |
| Encoding | System default |
| Format | Comma-separated text with Python-style tuple brackets |
| Source | Network share (Raspberry Pi) |

Input Field Definitions

| Index | Field Name | Type | Unit | Notes |
| --- | --- | --- | --- | --- |
| 0 | time | String | — | Observation timestamp |
| 1 | icao | String | — | ICAO address (aircraft ID) |
| 2 | altitude | double | feet | Pressure altitude |
| 3 | latitude | double | deg | Latitude |
| 4 | longitude | double | deg | Longitude |
| 5 | trackangle | double | deg | Ground track angle |
| 6 | groundspeed | double | knot | Ground speed |
| 7 | trueairspeed | double | knot | True airspeed (TAS) |
| 8 | heading | double | deg | Magnetic heading |
| 9 | (unused) | — | — | — |
| 10 | mach | double | — | Mach number |

## 2.2 Output File

| Item | Details |
| --- | --- |
| File path | D:\MyDocuments\adsb\modeSweather\{yyyyMMddHH}modeS_weather.csv |
| Encoding | System default |
| Format | CSV (no header row) |

Output Field Definitions

| Order | Field Name | Type | Unit | Notes |
| --- | --- | --- | --- | --- |
| 1 | time | String | — | Observation timestamp |
| 2 | icao | String | — | ICAO address |
| 3 | altitude | double | m | Altitude (converted from feet) |
| 4 | trackangle | double | deg | Ground track angle |
| 5 | gs_ms | double | m/s | Ground speed (converted from knots) |
| 6 | heading | double | deg | Magnetic heading |
| 7 | tas_ms | double | m/s | True airspeed (converted from knots) |
| 8 | temperature | double | °C | Outside air temperature (derived) |
| 9 | wSpeed | double | m/s | Wind speed (derived) |
| 10 | wAngle | double | deg | Wind direction (derived) |

# 3. Constants

| Constant | Value | Description |
| --- | --- | --- |
| MAGNETIC_DECLINATION | 7.64 | Magnetic declination in degrees for the Japan region. Used in wind direction calculation. |
| INVALID_VALUE | -999.0 | Sentinel value returned when a field is missing or unparseable. Falls outside the airspace filter bounds and is thus automatically excluded. |

# 4. Processing Logic

## 4.1 main Method

Entry point. Executes the following steps in order.

4.1.1 File Name Generation

The current date and time are formatted as yyyyMMddHH and embedded in both the input and output file paths.

4.1.2 Data Filtering

Valid records are selected by two sequential filters.

Stage 1 — Airspace filter (latitude / longitude)

| Parameter | Condition |
| --- | --- |
| latitude | >= 34.75 and <= 36.5 |
| longitude | >= 137.5 and <= 140.5 |

Note: INVALID_VALUE (−999.0) lies outside these bounds and is therefore rejected automatically at this stage.

Stage 2 — Valid-value filter

| Field | Condition |
| --- | --- |
| heading | > 0 |
| trueairspeed | > 0 |
| groundspeed | > 0 |
| mach | > 0 |

4.1.3 Unit Conversions

| Field | Source unit | Target unit | Factor |
| --- | --- | --- | --- |
| altitude | feet | m | × 0.3048 |
| groundspeed | knot | m/s | × 0.514 |
| trueairspeed | knot | m/s | × 0.514 |

# 5. Method Specifications

## 5.1 parseDoubleOrDefault

| Item | Details |
| --- | --- |
| Signature | private static double parseDoubleOrDefault(String str, double defaultValue) |
| Purpose | Safely converts a String to double |
| Returns | Parsed value on success; defaultValue if the input is null, "None", empty, or not parseable as a number |

## 5.2 temperature — Outside Air Temperature

| Item | Details |
| --- | --- |
| Signature | public static double temperature(double trueAirSpeed, double mach) |
| Purpose | Derives outside air temperature (°C) from TAS and Mach number |
| Returns | Outside air temperature in °C. Returns 0 when mach == 0 (division-by-zero guard). |

Formula

| Variable | Value / Expression | Description |
| --- | --- | --- |
| k | 1.403 | Heat capacity ratio of air |
| M | 28.966 × 10⁻³ | Molar mass of air [kg/mol] |
| R | 8.314472 | Universal gas constant [J/(mol·K)] |
| soundSpeed | TAS / Mach | Speed of sound [m/s] |
| T (°C) | soundSpeed² × (M / (k × R)) − 273.15 | Outside air temperature [°C] |

## 5.3 wind_x — East–West Wind Component

| Item | Details |
| --- | --- |
| Signature | public static double wind_x(double trackangle, double groundspeed, double heading, double trueairspeed) |
| Purpose | Computes the east–west component of the wind vector |
| Returns | East–west wind speed [m/s]; positive = eastward component |

Formula

| Variable | Expression | Description |
| --- | --- | --- |
| ground_dir | π/2 − trackangle [rad] | Ground track in math coordinates |
| ground_x | groundspeed × cos(ground_dir) | East–west component of ground velocity |
| air_dir | π/2 − heading [rad] + MAGNETIC_DECLINATION [rad] | Airspeed direction (magnetic declination corrected) |
| air_x | trueairspeed × cos(air_dir) | East–west component of air velocity |
| wind_x | ground_x − air_x | East–west wind component |

## 5.4 wind_y — North–South Wind Component

| Item | Details |
| --- | --- |
| Signature | public static double wind_y(double trackangle, double groundspeed, double heading, double trueairspeed) |
| Purpose | Computes the north–south component of the wind vector |
| Returns | North–south wind speed [m/s]; positive = northward component |

The formula mirrors wind_x with sin replacing cos to obtain the north–south component.

## 5.5 windspeed — Wind Speed

| Item | Details |
| --- | --- |
| Signature | public static double windspeed(double speedx, double speedy) |
| Formula | sqrt(speedx² + speedy²) |
| Returns | Wind speed [m/s] |

## 5.6 windangle — Wind Direction

| Item | Details |
| --- | --- |
| Signature | public static double windangle(double speedx, double speedy) |
| Formula | (π/2 − atan2(speedy, speedx) + 3π) mod 2π, converted to degrees |
| Returns | Wind direction [degrees], 0–360°, meteorological convention (from-direction) |

# 6. Error Handling

| Situation | Handling |
| --- | --- |
| Input file not found or unreadable | Exception caught in the catch block; error message and the offending line are printed to the console |
| Fewer than 11 fields in a line | Line is skipped (continue) |
| Field value is "None" or empty | parseDoubleOrDefault returns INVALID_VALUE (−999.0) or 0.0 depending on the call site |
| Field cannot be parsed as a number | NumberFormatException is caught; defaultValue is returned |
| mach == 0 in temperature() | Returns 0 to prevent division by zero (in practice this is guarded by the valid-value filter before the method is called) |

# 7. Constraints and Notes

- The magnetic declination value (7.64°) is hard-coded. This value changes over time and varies by location; periodic review is recommended.

- Output values are written as raw Java doubles, which may produce many decimal places (e.g. 15.000000000000002). Consider applying String.format("%.2f", value) if cleaner output is desired.

- Input field index [9] is unused. Verify against the input specification if the upstream data format changes.

- File encoding is determined by the system default and may cause mojibake in environments where the default encoding differs from the file's actual encoding.

- The network share path (\\192.168.1.zz\...) must be updated to match the actual IP address of the Raspberry Pi in the deployment environment.

# 8. Acknowledgements

The development of this program was greatly informed by research published by Dr. Hiroshi Ishikawa of the Natural Research Laboratory.

| Item | Details |
| --- | --- |
| Reference | "Upper-Air Weather Observation Using Raspberry Pi and Mode S" |
| Author | Dr. Hiroshi Ishikawa — Natural Research Laboratory (Hino, Tokyo) |
| URL | https://www.ishikawa-lab.com/RasPi_ModeS.html |
| Published | 19 June 2024 |

The following aspects of the present program draw directly from that work:

- The physical model for outside air temperature derivation — inverting the speed-of-sound formula using TAS and Mach number — and the associated constants (heat capacity ratio k = 1.403, molar mass M = 28.966 × 10⁻³ kg/mol, gas constant R = 8.314472 J/(mol·K))

- The wind vector calculation method: deriving wind from the vector difference between ground velocity and air velocity

- The magnetic declination correction concept and the adopted value of 7.64°

- The input data structure: Mode S field definitions and the concept of correlating ADS-B, BDS50, and BDS60 messages by ICAO address

- The airspace filter bounds: latitude 34.75°–36.5° N, longitude 137.5°–140.5° E

The PyModeS library (Junzi Sun, Delft University of Technology) was also referenced through the above work for its role in decoding raw Mode S messages on the Raspberry Pi side.

The original design concept of this program belongs to Dr. Ishikawa. The Java implementation documented here is a reconstruction and refinement of that concept.

Dr. Hiroshi Ishikawa (1942–2025)

Doctor of Engineering. After completing graduate studies at Waseda University,

he joined Nippon Telegraph and Telephone Public Corporation (now NTT),

where he served as Director of Research, Head of Network Operations, and Corporate Officer.

Honorary Member of the Institute of Electronics, Information and Communication Engineers (IEICE).

In retirement, he continued to pursue his passion for weather observation and electronics,

independently developing and sharing an upper-air weather observation system based on Mode S.

He passed away on 12 August 2025 at the age of 83. We offer our deepest condolences.